- 寄存器修改

a)新增和Remove 掉R2C的寄存器如下表，其他如 enable 和clk gate disable 寄存器保持原样未作改动。

b)degamma和 gamma的寄存器是shadow还是nonshadow 受到

dcFeature_BE_Gamma_Table_Ver控制

c)其中每个原本带有cnt的寄存器在改为Nonshadow后去除cnt，改为添加一个dcreg*Index的寄存器，原本寄存器本身的读写属性不变，index的寄存器均为write only。

d)dcregAIndex寄存器决定了本次读写的A寄存器的指针，Reset值为0,在每次读/写后会自加1，到达A寄存器的max count 后会恢复为0.

- 写寄存器流程：

由于改为non shadow寄存器，因为写这些寄存器的时机应和其他non shadow寄存器保持一致。

写某寄存器(如dcregA)前推荐先配置Index寄存器，如为第一次配置应配Index为0，再向dcregA所在寄存器地址写入，第一次写入为dcregA_Index，第二次写入为dcregA_Index+1，以此类推。

- 读寄存器流程：

读某寄存器(如dcregA)前推荐先配置Index寄存器，如为第一次配置应配Index为0，再向dcregA所在寄存器地址读取，第一次读取的值为dcregA_Index，第二次读取的值为dcregA_Index+1，以此类推。

| Register Naming | Register Module | Field | Cnt | Operation | Feature Table Define | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| dcregShPanel[X]DeGammaSegmentCount | DpuBePostProcessorPanel[X] | Value[3:0] |  | remove r2c | dcFeature_BE_outCtrl[X]_PostPro_CC_DeGamma_EN && dcFeature_BE_Gamma_Table_Ver == "Shadow" |  |
| dcregShPanel[X]DeGammaSegmentPoint1 |  | Value[11:0] | 15 |  |  |  |
| dcregShPanel[X]DeGammaSegmentStep |  | Value[12:0] | 16 |  |  |  |
| dcregShPanel[X]DeGammaData |  | Value[17:0] | 129 |  |  |  |
| dcregShPanel[X]GammaSegmentCount |  | Value[3:0] |  |  | dcFeature_BE_outCtrl[X]_PostPro_CC_Gamma_Prim_EN && dcFeature_BE_Gamma_Table_Ver == "Shadow" |  |
| dcregShPanel[X]GammaSegmentPoint1 |  | Value[16:0] | 15 |  |  |  |
| dcregShPanel[X]GammaSegmentStep |  | Value[16:0] | 16 |  |  |  |
| dcregShPanel[X]GammaGreenSegmentCount |  | Value[3:0] |  |  |  |  |
| dcregShPanel[X]GammaGreenSegmentPoint1 |  | Value[16:0] | 15 |  |  |  |
| dcregShPanel[X]GammaGreenSegmentStep |  | Value[16:0] | 16 |  |  |  |
| dcregShPanel[X]GammaBlueSegmentCount |  | Value[3:0] |  |  |  |  |
| dcregShPanel[X]GammaBlueSegmentPoint1 |  | Value[16:0] | 15 |  |  |  |
| dcregShPanel[X]GammaBlueSegmentStep |  | Value[16:0] | 16 |  |  |  |
| dcregShPanel[X]GammaRedData |  | Value[13:0] | 257 |  |  | write only |
| dcregShPanel[X]GammaGreenData |  | Value[13:0] | 257 |  |  | write only |
| dcregShPanel[X]GammaBlueData |  | Value[13:0] | 257 |  |  | write only |

| dcregPanel[X]DeGammaSegmentCount | DpuBePostProcessorPanel[X] | Value[3:0] |  | add reg r2c | dcFeature_BE_outCtrl[X]_PostPro_CC_DeGamma_EN && dcFeature_BE_Gamma_Table_Ver == "Non_shadow" |  |
| --- | --- | --- | --- | --- | --- | --- |
| dcregPanel[X]DeGammaSegmentPoint1Index |  | Value[3:0] |  |  |  | write only |
| dcregPanel[X]DeGammaSegmentPoint1 |  | Value[11:0] |  |  |  | Max cnt15 |
| dcregPanel[X]DeGammaSegmentStepIndex |  | Value[3:0] |  |  |  | write only |
| dcregPanel[X]DeGammaSegmentStep |  | Value[12:0] |  |  |  | Max cnt16 |
| dcregPanel[X]DeGammaDataIndex |  | Value[7:0] |  |  |  | write only |
| dcregPanel[X]DeGammaData |  | Value[17:0] |  |  |  | Max cnt129 |
| dcregPanel[X]GammaSegmentCount |  | Value[3:0] |  |  | dcFeature_BE_outCtrl[X]_PostPro_CC_Gamma_Prim_EN && dcFeature_BE_Gamma_Table_Ver == "Non_shadow" |  |
| dcregPanel[X]GammaSegmentPoint1Index |  | Value[3:0] |  |  |  | write only |
| dcregPanel[X]GammaSegmentPoint1 |  | Value[16:0] |  |  |  | Max cnt15 |
| dcregPanel[X]GammaSegmentStepIndex |  | Value[3:0] |  |  |  | write only |
| dcregPanel[X]GammaSegmentStep |  | Value[16:0] |  |  |  | Max cnt16 |
| dcregPanel[X]GammaGreenSegmentCount |  | Value[3:0] |  |  |  |  |
| dcregPanel[X]GammaGreenSegmentPoint1Index |  | Value[3:0] |  |  |  | write only |
| dcregPanel[X]GammaGreenSegmentPoint1 |  | Value[16:0] |  |  |  | Max cnt15 |
| dcregPanel[X]GammaGreenSegmentStepIndex |  | Value[3:0] |  |  |  | write only |
| dcregPanel[X]GammaGreenSegmentStep |  | Value[16:0] |  |  |  | Max cnt16 |
| dcregPanel[X]GammaBlueSegmentCount |  | Value[3:0] |  |  |  | Max cnt15 |
| dcregPanel[X]GammaBlueSegmentPoint1Index |  | Value[3:0] |  |  |  | write only |
| dcregPanel[X]GammaBlueSegmentPoint1 |  | Value[16:0] |  |  |  | Max cnt15 |
| dcregPanel[X]GammaBlueSegmentStepIndex |  | Value[3:0] |  |  |  | write only |
| dcregPanel[X]GammaBlueSegmentStep |  | Value[16:0] |  |  |  | Max cnt16 |
| dcregPanel[X]GammaRedDataIndex |  | Value[8:0] |  |  |  | write only |
| dcregPanel[X]GammaRedData |  | Value[13:0] |  |  |  | write only Max cnt257 |
| dcregPanel[X]GammaGreenDataIndex |  | Value[8:0] |  |  |  | write only |
| dcregPanel[X]GammaGreenData |  | Value[13:0] |  |  |  | write only Max cnt257 |
| dcregPanel[X]GammaBlueDataIndex |  | Value[8:0] |  |  |  | write only |
| dcregPanel[X]GammaBlueData |  | Value[13:0] |  |  |  | write only Max cnt257 |